lEONARD WASSENAAR

25948083

ITRW 322

PT5 Activity 6.7.5: Subnet and Router

Task 1: Subnet the Address Space.

Step 1: Examine the network requirements.

Step 2: Consider the following questions when creating your network design.

How many subnets are needed for this network? 3

What is the subnet mask for this network in dotted decimal format? 255.255.255.224

What is the subnet mask for the network in slash format? /27

How many usable hosts are there per subnet? 30

Step 3: Assign subnetwork addresses to the Topology Diagram.

1. 192.168.1.32/27

2. 192.168.1.64/27

3. 192.168.1.96/27

Task 2: Determine Interface Addresses.

Step 1: Assign appropriate addresses to the device interfaces.

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | Fa0/0 | 192.168.1.33 | 255.255.255.224 |  |
|  | S0/0/0 | 192.168.1.65 | 255.255.255.224 |  |
| R2 | Fa0/0 | 192.168.1.97 | 255.255.255.224 |  |
|  | S0/0/0 | 192.168.1.94 | 255.255.255.224 |  |
| PC1 | NIC | 192.168.1.62 | 255.255.255.224 | 192.168.1.33 |
| PC2 | NIC | 192.168.1.126 | 255.255.255.224 | 192.168.1.97 |

Task 3: Configure the Serial and FastEthernet Addresses

Step 1: Configure the router interface

Step 2: Configure the PC interfaces.

Task 4: Verify the Configurations

- From the host attached to R1, is it possible to ping the default gateway? Yes

- From the host attached to R2, is it possible to ping the default gateway? Yes

- From the router R1, is it possible to ping the Serial 0/0/0 interface of R2? Yes

- From the router R2, is it possible to ping the Serial 0/0/0 interface of R1? Yes

Task5: Reflection

The router must be configured fist, either static or dynamic before we can ping, example between PC1 and PC2